RCF introduces the new EVOX J

True 3-way active column systems for portable, high-definition sound

Reggio Emilia, Italy — RCF expands its EVOX lineup with a family of three active column systems built around a true 3-way electroacoustic design, a new amplification platform, and a clean, neutral aesthetic. All models are available in black or white. Drawing on more than a decade of EVOX adoption in DJ, live performance, and rental applications, EVOX J is a ground-up evolution focused on higher headroom, clearer vocal projection, and faster deployment.

At the heart of EVOX J is a dedicated high-frequency section: a 1.75” neodymium compression driver (Kapton dome) loaded by RCF’s True Resistive Waveguide (TRW), engineered to deliver constant 120° × 40° coverage and designed to reduce typical horn harshness. The midrange is handled by an array of 3” neodymium cone transducers that integrates the HF section to provide the focused “single-source” feel associated with point-source systems.

Low frequencies come from a high-power 12” woofer in a bass-reflex enclosure. A new generation of Class-D amplification delivers 2100 W (J9/JMIX9) or 3500 W (J11), providing significantly more headroom than previous architectures. The system is managed by onboard processing that includes FiRPHASE (0º linear-phase response with minimal latency), Bass Motion Control (extended low-end without instability), and protection features.

The range includes:

- EVOX J9: 130 dB Max SPL, 2100 W, 6 × 3” mid array, three EQ presets (Live/Linear, Club, Voice)

- EVOX JMIX9: J9 acoustic platform plus an integrated 8-channel stereo digital mixer with 2.4” color touch display, THAT® mic preamps, Bluetooth Audio, per-channel processing, scene recall, audio ducker, reverb and JMixRemote app control via Bluetooth BLE (with optional security PIN).

- EVOX J11: 132 dB Max SPL, 3500 W, 12 × 3” mid array, four EQ presets (Live, Linear, Club, Voice) and PowerCON TRUE1-TOP for more demanding professional use.

For extended low-frequency impact, EVOX J systems can be paired with compatible RCF subwoofers using ACE integration presets (i.e. SUB AX series), aligning delay and phase for optimized summation and extreme bass impact.

###

Photo file 1: EVOX_J9_W_B.JPG

Photo caption 1: RCF EVOX J9 in white and black

Photo file 2: EVOX_J11_W_B.JPG

Photo caption 2: RCF EVOX J11 in white and black

RCF – Company Profile

RCF is a leading brand in professional audio equipment, established in Italy in 1949. Covering all aspects of the audio chain, including events and concerts, recording, public address, broadcast, and portable audio, RCF boasts 75 years of history at the forefront of technology and materials research. RCF provides tools, documentation, technical support, and training to help professionals deliver optimized sound to their audiences. With a comprehensive product catalog that meets the most demanding requirements of musicians, sound engineers, architects, and system integrators, RCF supports its customers through offices in the USA, France, Germany, Italy, Spain, the UK and a network of over 130 trusted distributors worldwide.